# Biological methane potential of pig manure from different dietary crude protein levels in Vietnam

By Tran Thi Thanh Thao

Vietnam National Institute of Animal and Veterinary Sciences.

1. Background and rationale

Livestock production is a major contributor to global greenhouse gas emissions, particularly methane (CH₄) generated during manure management (Gerber et al., 2013). Methane is produced when organic matter in stored manure undergoes anaerobic decomposition by microbial communities (IPCC, 2006). Because methane has a significantly higher global warming potential than carbon dioxide, accurate estimation of methane emissions from manure is important for improving greenhouse gas inventories and developing mitigation strategies (IPCC, 2019).

In Vietnam, pig production has increased substantially in recent decades, According to data from the General Statistics Office, as of April 2024, the total pig population nationwide reached approximately 27.5 million head (GSO, 2026). This expansion has resulted in increased volumes of animal manure, which are often managed through storage systems such as lagoons, pits, or biogas digesters. The amount of methane generated from manure depends on several factors, including manure composition, microbial activity, and manure management conditions (IPCC, 2019). Among these factors, animal diet plays an important role because it influences manure characteristics such as organic matter content, nitrogen concentration, and degradability (Hristov et al., 2013).

Methane from pig manure is a priority source for improving Vietnam’s livestock greenhouse gas inventory. In the national inventory, methane from pig manure was the main source of manure management emissions, accounting for 92.6% in 1994 and 90.7% in 2022. Over the same period, methane emissions from pig manure increased from 303.12 Gg CH4 to 749.21 Gg CH4, largely due to the expansion of the pig population. The recent Tier 2 work also shows that pig manure remains a major source in the updated inventory, with manure CH4 reaching 112.37 Gg in 2022. These figures show that improving methane estimates from pig manure is essential for Vietnam’s national inventory accuracy, mitigation planning, and climate reporting (Le Thi Thanh Huyen et al., 2025).

The Biological Methane Potential (BMP) assay is a widely used laboratory method for determining the maximum methane production potential (Bo) of organic substrates under controlled anaerobic conditions (Holliger et al., 2016). BMP tests simulate anaerobic digestion processes and provide important information about methane yield and substrate degradability (Raposo et al., 2011). Because Bo is a core parameter in the IPCC Tier 2 method for estimating methane emissions from manure management, country-specific measurement of Bo from Vietnamese pig manure is needed. The Tier 2 equation directly links methane emission factors to volatile solids, manure management system shares, methane conversion factors, and Bo. As a result, uncertainty in Bo directly affects the accuracy of methane emission factors and the reliability of national estimates. Direct measurement of Bo from Vietnam pig manure will therefore provide a stronger scientific basis than continued reliance on default values that may not reflect Vietnamese diets, manure characteristics, production systems, and climate conditions.

Despite the importance of pig production in Vietnam, limited experimental data are available on the methane production potential of pig manure under different feeding systems. Generating BMP data from locally relevant diets will help improve methane emission factor estimates and support climate change mitigation strategies in the livestock sector.

2. Research objectives

Main objective:

The main objective of this study is to quantify the biological methane potential, Bo of pig manure from pigs under different dietary treatments, while strengthening national capacity for Bo measurement and supporting the generation of country specific data for Vietnam’s Tier 2 GHG inventory.

Specific objectives:

- Determine the chemical composition and biological methane potential, Bo, of pig manure under four dietary treatments.

- Compare methane yields across the dietary treatments.

- Evaluate the relationship between manure chemical composition and methane production from manure.

- Generate experimental data to support the development of country specific methane emission factors for Vietnam’s Tier 2 GHG inventory.

- Build technical capacity in Vietnam on Bo laboratory methods, data processing, and interpretation, and support the establishment of standardized Bo measurement procedures for pig manure.

- 3. Research hypothesis

It is hypothesized that pig diets with different crude protein levels will produce manure with different chemical composition and biological biological methane potentials. Variations in chemical composition such as nutrient composition and organic matter degradability are expected to influence microbial activity during anaerobic digestion, resulting in differences in biological methane production between treatments.

4. Methodology

4.1 Experimental design

The study will employ a batch anaerobic digestion experiment to determine the biological methane potential of pig manure from four dietary treatments. Manure samples will be collected from pigs that were fed diets with different crude protein levels as part of an existing feeding experiment.

Each treatment will be tested in triplicate BMP reactors. In addition, blank controls (inoculum only) and positive controls (cellulose) will be included to ensure experimental validity.

4.2. Manure collection and storage

Manure samples were daily collected from pigs in two periods (75kg- 100kg and 100kg- market weight) under each dietary treatment, which homogenized and stored at −20°C prior to laboratory analysis (table.1). Before BMP incubation, samples will be thawed at 4°C and mixed thoroughly to ensure representative sampling.

Table 1. Percentage of CP in pig diet in different stage

| Stage Treatment | 75-100kg | 100kg-market weight |
| --- | --- | --- |
| 1 | 15,0 | 14,0 |
| 2 | 14,0 | 13,0 |
| 3 | 13,0 | 12,0 |
| 4 | 12,0 | 11,0 |

4.3. Inoculum preparation

Anaerobic inoculum will be obtained from an active biogas digester treating livestock manure. The inoculum will be sieved to remove large particles and pre-incubated at 37°C for 7-14 days to reduce residual biogas production before use in the BMP assay.

4.4. BMP incubation setup

BMP tests will be conducted using sealed serum bottles as batch anaerobic reactors. Each bottle will have a total volume of approximately 1L with a working volume of 600–800 mL. The substrate-to-inoculum ratio will be adjusted to 1:2 based on volatile solids (VS) content to avoid microbial inhibition.

Bottles will be flushed with nitrogen gas to establish anaerobic conditions and sealed with butyl rubber stoppers. Reactors will be incubated at 37°C in a temperature-controlled incubator for approximately 30–40 days. Bottles will be manually shaken daily to ensure proper mixing.

4.5. Gas measurement and analysis

Biogas production will be monitored periodically using a pressure-based method. Headspace pressure in each bottle will be measured using a digital manometer. Gas samples will be collected using gas-tight syringes and analyzed for methane concentration using gas chromatography (GC).

Methane production will be calculated based on gas pressure and methane concentration, and results will be expressed as milliliters of methane per gram of volatile solids added (mL CH₄ g⁻¹ VS). Methane production from blank reactors will be subtracted to obtain net methane yields.

4.6. Chemical analysis

The following parameters will be measured for both manure and inoculum samples: total solids (TS), crude protein and fat content, volatile solids (VS), and pH. Standard laboratory methods will be used for these analyses.

4.7. Data analysis

Methane production curves will be analyzed using kinetic models such as the modified Gompertz model to estimate maximum methane potential and production rates. Statistical comparisons between treatments will be conducted using analysis of variance (ANOVA) followed by appropriate post-hoc tests.

- The policy relevance

Vietnam has already received support to move from Tier 1 toward Tier 2 livestock greenhouse gas inventories through the project “Capacity Development for Inventory, Measurement, Reporting and Verification of Livestock GHG Emissions in Viet Nam,” funded by the New Zealand Climate Smart Agriculture Initiative in support of the Global Research Alliance on Agricultural Greenhouse Gases. That project built capacity for more than 50 Vietnamese researchers and officials, developed country specific Tier 2 emission factors for cattle and pigs, and established tools for inventory compilation and MRV. Importantly, the final report explicitly recommended further research to develop country specific inputs for pig sub categories from different farm types and climate zones, including DE values, Bo values, emission factors, and Nex values. The proposed Bo determination study therefore responds directly to an identified national research need and will help close one of the remaining gaps for strengthening Vietnam’s Tier 2 pig manure inventory.

6. Expected outcomes

This study is expected to generate generate measured Bo values for pig manure derived from different dietary treatments. The results will improve understanding of how diet composition affects methane production from manure.

The project is expected to generate measured Bo values for representative pig manure systems in Vietnam, establish a documented laboratory protocol for  determination under Vietnamese conditions, strengthen national capacity to run and interpret Bo assays, and provide recommendations for integrating the resulting country specific Bo values into Vietnam’s Tier 2 inventory and MRV framework for the pig sector. The results may also provide useful information for optimizing manure management and biogas production systems.

7. Capacity Building

Through this fellowship, Thao was exposed to international best practice in laboratory testing, data analysis, and interpretation at Mazingira’s biogas research facility. This project will convert that training into practical national capacity in Vietnam by helping establish the test locally, strengthening laboratory procedures and analytical quality control, and supporting harmonized measurement protocols for Vietnamese pig manure across farm types and climate zones. In this way, the project will contribute not only new data, but also long term technical capacity for VIAVS and national partners to generate country specific manure methane parameters for research, inventory improvement, and mitigation planning.

This capacity building objective is well aligned with recent national efforts that already supported training on manure emission measurement equipment, gas chromatography systems, and standard operating procedures for manure emission studies. The proposed work would extend that progress by adding a missing laboratory component that is directly relevant to pig manure methane estimation in Vietnam..

8. Proposed timeline

Week 1–2: Sample preparation and laboratory setup.

Week 3: BMP experiment initiation.

Week 4–7: Incubation and biogas monitoring.

Week 8: Data analysis and report preparation.

9. Reference

Gerber, P. J., Steinfeld, H., Henderson, B., Mottet, A., Opio, C., Dijkman, J., Falcucci, A., & Tempio, G. (2013). Tackling climate change through livestock: A global assessment of emissions and mitigation opportunities. Food and Agriculture Organization of the United Nations (FAO). https://www.fao.org/3/i3437e/i3437e.pdf

General Statistics Office of Vietnam. (2026). National statistics database. https://www.nso.gov.vn/en/homepage/

Holliger, C., Alves, M., Andrade, D., Angelidaki, I., Astals, S., Baier, U., Bougrier, C., Buffière, P., Carballa, M., de Wilde, V., Van Eekert, M., & Van Lier, J. (2016). Towards a standardization of biomethane potential tests. Water Science and Technology, 74(11), 2515–2522. https://doi.org/10.2166/wst.2016.336

Hristov, A. N., Oh, J., Firkins, J. L., Dijkstra, J., Kebreab, E., Waghorn, G., Makkar, H. P. S., Adesogan, A. T., Yang, W., Lee, C., Gerber, P. J., Henderson, B., & Tricarico, J. M. (2013). Mitigation of greenhouse gas emissions in livestock production: A review of technical options for non-CO₂ emissions. Food and Agriculture Organization of the United Nations (FAO). https://www.fao.org/3/i3288e/i3288e.pdf

IPCC. (2006). 2006 IPCC guidelines for national greenhouse gas inventories. Intergovernmental Panel on Climate Change. https://www.ipcc-nggip.iges.or.jp/public/2006gl/

IPCC. (2019). 2019 refinement to the 2006 IPCC guidelines for national greenhouse gas inventories. Intergovernmental Panel on Climate Change. https://www.ipcc-nggip.iges.or.jp/public/2019rf/

Le Thi Thanh Huyen, Tran Thi Bich Ngoc, Nguyen Thanh Trung, Chu Manh Thang, Tran Viet Phuong, Tran Thi Thanh Thao, Dao Thi Binh An, Pham Thi Thanh Huong, & Pham Doan Lan. (2025). Capacity development for inventory, measurement, reporting and verification of livestock GHG emissions in Viet Nam: Final project report. National Institute of Animal Science.

Mazingira Centre, International Livestock Research Institute (ILRI). (2025). Biological methane potential (BMP) standard operating procedure. International Livestock Research Institute.

Raposo, F., De la Rubia, M. A., Fernández-Cegrí, V., & Borja, R. (2012). Anaerobic digestion of solid organic substrates in batch mode: An overview relating to methane yields and experimental procedures. Renewable and Sustainable Energy Reviews, 16(1), 861–877. https://doi.org/10.1016/j.rser.2011.09.008